%% Clear environment

warning off

close all

clear

clc

%% Parameter settings

numTrees = 200;

maxTreeDepth = 3;

numRuns = 10;

%% Load data

data = readtable('cejing.xlsx');

X = data{1:47, 1:10};

y = data{1:47, 11};

%% Feature labels

featureNames = {'Depth', 'GR', 'AC', 'SP', 'RD', 'T', 'RS', 'CNL', 'DEN', 'MSFL'};

%% Cumulative feature importance calculation

cumulativeFeatureImportance = zeros(1, 10);

for run = 1:numRuns

currentSeed = 42 + run * 12;

rng(currentSeed, 'twister');

treeOptions = statset('TreeBagger');

treeOptions.MaxDepth = maxTreeDepth;

treeOptions.Streams = RandStream('mt19937ar','Seed', currentSeed);

ens = TreeBagger(numTrees, X, y, ...

'Method', 'regression', ...

'OOBPredictorImportance', 'on', ...

'Options', treeOptions, ...

'NumPredictorsToSample', 'all');

featureImportance = ens.OOBPermutedVarDeltaError;

minVal = min(featureImportance);

if minVal < 0

featureImportance = featureImportance - minVal;

end

cumulativeFeatureImportance = cumulativeFeatureImportance + featureImportance;

end

%% Average feature importance

featureImportanceAvg = cumulativeFeatureImportance / numRuns;

%% Normalized contribution rate

normalizedImportance = featureImportanceAvg / sum(featureImportanceAvg);

%% Sort features by importance

[sortedImportance, sortedIdx] = sort(normalizedImportance, 'descend');